This document is intended for use when performing BMC firmware update with preserve and restore options, follow the instructions below to avoid the configuration restore issue.

Scope

| Platform | 13th Gen Platforms |
| --- | --- |
| Tool | Supermicro Update Manager (SUM) v2.13.0 |

Instructions

- Run the following commands and option to update the BMC firmware to the new version on the target node. Command mode; Command usage. Inband; ./sum -c UpdateBmc --file bmc.bin. OOB; ./sum -i <BMC IP> -u <BMC username> -p <BMC password> -c UpdateBmc --file bmc.bin. As shown in the example, executed with OOB mode. - After updating the BMC firmware to the new version on the target node, download the current BMC configuration and save it as an .xml file, such as bmccfg.xml in the example. Command mode; Command usage. Inband; ./sum -c GetBmcCfg --file bmccfg.xml --overwrite. OOB; ./sum -i <BMC IP> -u <BMC username> -p <BMC password> -c GetBmcCfg --file bmccfg.xml --overwrite. As shown in the example, executed with OOB mode. - Edit the downloaded .xml file and modify the BMC Connection Interface settings from "Failover" to "Dedicated", as shown in the example. - Save and apply the configured BMC configuration file back to the BMC of the target node. Command mode; Command usage. Inband; ./sum -c ChangeBmcCfg --file bmccfg.xml. OOB; ./sum -i <BMC IP> -u <BMC username> -p <BMC password> -c ChangeBmcCfg --file bmccfg.xml. As shown in the example, executed with OOB mode.