This is the SOP of using intel FPT under “EFI shell”

- Boot into EFI shell (Need a bootable key with Shell.efi)

- Type mm FDAE05B8 4000200 –w 4 (in order to use FPT to flash the ROM)

- Type mm CF9 0e –w 1 –IO (in order to do a warm reboot)

- Case1: Flash Bios under the shell

  - After reboot, boot into the EFI shell.

  - Boot into the removable USB key where you put the intel Fpt.efi and update_image(in this case fs1:)

  - Then type fpt –f R01-B12.bin  (the version)

  - The flash should start.

  - After the FPT Operation Successful.

  - Type mm cf9 06 –w 1 –IO (in order to do a cold reboot)

  - The BIOS and ME should been update successfully.

- Case 2: Flash the Bios under the Dos

  - After reboot, boot into the Dos

  - . Enter the file folder which have fpt.exe and R01-B12.bin

  - Type fpt –f R01-B12. bin (your version)

If you miss the step before (step 1-3) , you will find this situation.

  - After the FPT Operation Successful , and Reboot

  - The BIOS and ME should been update successfully.