This is the SOP of update BIOS for C7C232 with bootable USB flash

Notice: Please follow this SOP to upgrade you system BIOS .

Update BIOS with USB flash (Use *.ROM)

All files are in the folder named "EFI".

Follow the steps to update BIOS with bootable USB flash.

1. Copy the 3 files under the folder to the root folder of bootable USB flash.

2. Boot system by the bootable USB flash (SHELL Mode).

3. Update BIOS with the command "flash" and press enter key.

4. System will reboot when finishing the BIOS update.

Notice:

- If the BIOS downgrade from the BIOS Version after R01-B18 to R01-B18 previous version, It needs to set the setup item in the Boot Page

Boot mode Select ->dual

Because we add the FixedBootOrder module after R01-B18.